Arranged cleaned and dried 500 mL R.B. flask set up equipped with mechanical stir device at 25°C(30±5°C) under Argon atmosphere

Observation:

N/A

Date & Time:

28.10.2025 09:06 am

2

Operation:

Charged THF lot-1and BCP 2 material & DIPA in to RBF at RT under Argon atmosphere cool to 15±5°C

Observation:

Homogeneous brown color mass

Date & Time:

28.10.2025 09:29 am

3

Operation:

Temperature attained ,slowly added LDA in 2M THF solution in to the RM at 15±5°C during in 1hr 30min

Observation:

Brown color homogeneous mass

Date & Time:

28.10.2025 09:53 am

4

Operation:

Addition completed stir for 1 hour

Observation:

Dark blue color homogeneous mass

Date & Time:

28.10.2025 11:02 am

5

Operation:

Slowly added 3-chloro benzyl chloride in 2V THF lot-2 solution in to the RM at -10to -15° under Orgon

Observation:

Dark blue color homogeneous mass

Date & Time:

28.10.2025 12:05 pm

6

Operation:

Addition completed , stir 1 hour at -10to -15° under Orgon

Observation:

Brown color homogeneous mass

Date & Time:

28.10.2025 02:05 pm

7

Operation:

Check the TLC reaction 55% above conversion observed raised temperature to 0-5°c stir for 1 hour

Observation:

Brown color homogeneous mass

Date & Time:

28.10.2025 03:14 pm

8

Operation:

Check the TLC reaction 55% above conversion observed , started workup

Observation:

Brown color homogeneous mass

Date & Time:

28.10.2025 04:37 pm

Work-up

9

Operation:

Slowly added (ammonium chloride+ Dm water lot-1) solution drop wised addition , stir for 20 mins at RT

Observation:

Lite brown color mass observed

Date & Time:

28.10.2025 05:07 pm

10

Operation:

Separate the layers, Aq layer exacted with toluene stir for 20 mins

Observation:

Lite brown color mass observed

Date & Time:

28.10.2025 05:43 pm

11

Operation:

Separate layers, total organic layer washed with (sodium chloride +DM water lot-2) solution stir for 20 mins

Observation:

Lite brown color mass observed

Date & Time:

28.10.2025 06:24 pm

12

Operation:

Separate the layers , distilled organic layer under vacuum at 50°C , Co-distill with IPA lot-1

Observation:

Brown color mass observed

Date & Time:

28.10.2025 06:58 pm

13

Operation:

Distillation completed , residue obtained

Observation:

Brown color mass observed

Date & Time:

28.10.2025 07:36 pm

Isolation

14

Operation:

Taken above crude into RBF, charged IPA lot-2 , heat to 60°C ,stir for 1 hour

Observation:

Brown color mass observed

Date & Time:

28.10.2025 07:58 pm

15

Operation:

Cool the mass at RT under stirring

Observation:

Brown color mass observed

Date & Time:

28.10.2025 09:05 pm

16

Operation:

Temperature attained at 31°C

Observation:

Yellow color solid observed

Date & Time:

28.10.2025 09:55 pm